CS 1104 Computer Systems

Module 1

Boolean Logic

## Overview

Boolean Logic is the mathematical foundation that makes computers tick. Every calculation your computer performs, every decision a program makes, and every bit stored in memory is based on simple true/false values and a handful of logical operations. Understanding Boolean Logic is the first step to understanding how hardware and software really work.

## 1. What is Boolean Logic?

Boolean Logic, developed by mathematician George Boole in the 1800s, operates on two values:

- TRUE  (represented as 1)

- FALSE  (represented as 0)

In computers, these two values map perfectly onto electrical signals: a high voltage is 1 (true), and a low voltage is 0 (false). Everything a computer does is ultimately built from billions of these tiny 1/0 decisions.

## 2. The Three Basic Operations

There are three fundamental Boolean operations. All others are derived from these.

### 2.1  NOT  (Inversion)

NOT flips the value. If input is 1, output is 0, and vice versa.

Symbol:  ¬A  or  A'  or  ~A

| Input A | NOT A (Output) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

Example:  If A = "It is raining" then NOT A = "It is NOT raining".

### 2.2  AND  (Conjunction)

AND outputs 1 only when BOTH inputs are 1. Think of it as: "A and B must both be true."

Symbol:  A · B  or  A AND B  or  A ∧ B

| Input A | Input B | A AND B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Example:  A door unlocks only if you have the right key AND enter the correct PIN.

### 2.3  OR  (Disjunction)

OR outputs 1 when at least ONE input is 1.

Symbol:  A + B  or  A OR B  or  A ∨ B

| Input A | Input B | A OR B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Example:  A car alarm sounds if the door is opened OR the window is broken.

## 3. Derived Operations

By combining the basic three, we get more powerful gates used everywhere in hardware design.

### 3.1  NAND  (NOT AND)

NAND is the opposite of AND. It outputs 0 only when BOTH inputs are 1. NAND is special: you can build every other gate from NAND alone — making it "functionally complete".

| Input A | Input B | A NAND B |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

### 3.2  NOR  (NOT OR)

NOR is the opposite of OR. Outputs 1 only when BOTH inputs are 0.

| Input A | Input B | A NOR B |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

### 3.3  XOR  (Exclusive OR)

XOR outputs 1 when the inputs are DIFFERENT. It's the foundation of binary addition — you'll use this heavily in the next module.

| Input A | Input B | A XOR B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Memory trick: XOR = "one or the other, but not both."

## 4. Boolean Expressions

Boolean operations can be combined into expressions, just like math. Operator precedence (high to low):

- 1st:  NOT  (strongest binding)

- 2nd:  AND

- 3rd:  OR  (weakest binding)

Example expression:

F = A · B + ¬C

This means: F is true if (A AND B) is true, OR if C is false.

Evaluation with A=1, B=0, C=0:

¬C  =  ¬0  =  1

A · B  =  1 · 0  =  0

F  =  0 + 1  =  1

## 5. De Morgan's Laws

These two laws are essential for simplifying and transforming Boolean expressions:

| Law | Expression | Meaning |
| --- | --- | --- |
| Law 1 | ¬(A · B) = ¬A + ¬B | NOT(A AND B) = (NOT A) OR (NOT B) |
| Law 2 | ¬(A + B) = ¬A · ¬B | NOT(A OR B) = (NOT A) AND (NOT B) |

Practical use: If you don't have an AND gate but you have NOR gates, De Morgan's Laws let you rearrange the expression to build the same logic with what you have.

Verification example for Law 1 with A=1, B=0:

Left side:   ¬(1 · 0)  =  ¬0  =  1

Right side:  ¬1 + ¬0  =  0 + 1  =  1  ✓

## 6. Why This Matters for Computer Systems

Every component you'll study in this course is built on Boolean Logic:

- The ALU (Module 3) is a network of AND/OR/XOR/NOT gates.

- Memory cells use NAND/NOR latches to store a single bit.

- The CPU's control unit uses Boolean expressions to decode instructions.

- Compiler optimizations often reduce Boolean expressions to use fewer gates, saving power.

Understanding Boolean Logic now means you'll recognize these patterns throughout the course.

## Summary

| Operation | Symbol | Output is 1 when... |
| --- | --- | --- |
| NOT | ¬A | A is 0 |
| AND | A · B | Both A and B are 1 |
| OR | A + B | At least one is 1 |
| NAND | ¬(A·B) | Not both are 1 |
| NOR | ¬(A+B) | Both are 0 |
| XOR | A ⊕ B | A and B are different |

🧠 Module Quiz

1. What is the output of: NOT 1 AND (0 OR 1)?

2. Fill in the missing value in the AND truth table: 1 AND __ = 0

3. Using De Morgan's Law, rewrite ¬(A + B) without using NOT on a compound expression.

4. A security system unlocks if: the badge is valid AND (the PIN is correct OR the manager approves). Write this as a Boolean expression using A = badge valid, P = PIN correct, M = manager approves.

5. What operation outputs 1 only when its two inputs are DIFFERENT? Why is this useful in binary arithmetic? (Hint: think about adding 1 + 1)